Product Specifications

Low Pressure Rubber Flex Connector

- Provide low pressure rubber connectors as indicated on the contract drawings or as needed to compensate for thermal expansion or to eliminate the transmission of vibration and noise through the piping system.

- Products:

  - Low pressure rubber connector shall be Style 101® as manufactured by the Metraflex Co. Chicago, IL.

  - Materials.

    - Low pressure rubber flex connector shall be of unreinforced corrugated Neoprene.

    - Low pressure rubber flex connector clamps on to plain end pipe with stainless steel hose clamps.

  - Maximum operating conditions shall be 4.3 PSI at 120⁰ F.

- Execution.

  - Rubber joints shall be installed per manufacturer’s instructions and in accordance to Rubber joints FSA's "Technical Handbook: Non-Metallic Expansion Joints and Flexible Pipe Connectors."

  - Refer to catalog data for rubber expansion joints movement capabilities.