Sample Specifications for Wall Penetration Seal

Product Specifications

Metraseal - Mechanical Pipe Seal:

Pipe wall penetration seals to be of the modular link type. Seals shall consist of a series of interlocking, molded synthetic rubber links, with heavy-duty plastic pressure plates, and corrosion resistant nuts and bolts. Seals to be designed to provide a hydrostatic seal between the pipe and wall penetration. Seals shall be sized and selected per manufacturer recommendations. Mechanical pipe seals shall be fabricated of an EPDM elastomer for general service and a Nitrile/ Buna-N for hydrocarbon/petroleum based applications. Provide stainless steel hardware as required. Mechanical pipe seals shall be model “Metraseal” as manufactured by The Metraflex Company®, Chicago, IL.

28. Model “WS” - Wall Sleeve (steel):

Cast in place concrete wall sleeves to be fabricated from galvanized heavy wall welded or seamless carbon steel pipe. All sleeves to have a 2" wide, full perimeter water stop, welded on both sides. Wall sleeves shall be model “WSCSG” as manufactured by The Metraflex Company®, Chicago, IL.

29. Model “WS” - Wall Sleeve (plastic):

Cast in place concrete wall sleeves to be fabricated from Schedule 40 PVC pipe. All sleeves to have a 2" wide, full perimeter water stop, welded on both sides. Wall sleeves shall be model “WSPVC” as manufactured by The Metraflex Company®, Chicago, IL.